BUG DETECTION ON HOME PHONES

FIRST OF ALL TO TEST FOR BUGS, YOU NEED A VOM (MULTIMETER) THE HIGHER THE

IMPEDANCE THE BETTER (A DIGITAL WITH FET CIRCUITRY OR A VACUUM TUBE VOLT METER

IS THE BEST).

FIRST DISCONNECT THE PHONE LINE(S) AT BOTH ENDS.  UNDO THE PHONE INSTRUMENT

AND HOOK IT UP TO THE ENTRY POINT OF THE PHONE LINE FROM THE OUTSIDE WORLD (MA

BELL DOES NOT LIKE YOU CUT HER OFF COMPLETELY.)  THE SCHEME IS THE PHYSICALLY

ISOLATE YOUR HOUSE, APARTMENT, ETC. FROM THE OUTSIDE WORLD.  BUT BEFORE YOU DO

THIS MEASURE THE LINE VOLTAGE (IT SHOULD BE APPROXIMATELY 48 VOLTS).

NOW WITH THE WIRES DISCONNECTED AT BOTH ENDS, SET YOUR RESISTANCE SCALE TO A

HIGH READING AND MEASURE THE RESISTANCE OF THE PHONE LINE, IT SHOULD BE VERY

HIGH ON THE ORDER OF MILLION OHMS OR MORE, THIS IS THE NORMAL CONDITION, SINCE

YOU ARE MEASURING THE RESISTANCE OF AN OPEN CIRCUIT.  IF IT IS MUCH LESS, SAY

50-100KOHMS THEN YOU A DEVICE ON THE LINE THAT DOES NOT BELONG THERE, PROBABLY

A PARALLEL BUG.

NOW TWIST THE END OF THE DISCONNECTED WIRE AND GO TO THE OTHER END AND

MEASURE THE RESISTANCE OF THIS.  THIS RESISTANCE SHOULD BE ABOUT ONE OHM OR

TWO AT THE MOST IN A BIG HOUSE WITH A LOT OF PHONES.  IF IT IS MORE, THEN YOU

PROBABLY HAVE A SERIES BUG.

IF IN THE FIRST CASE, TAKING PARALLEL MEASUREMENTS USING A METER (NOT

LEDLLCD) AND YOU NOTICE A "KICK" IN THE NEEDLE, YOU PROBABLY HAVE A LINE TAP.

NOW IF YOU ALSO MAKE A MEASUREMENT WITH THE WIRE END TWISTED TOGETHER AND

YOU NOTICE THE RESISTANCE READS ABOUT 1-2KOHMS, THEN YOU MAY HAVE A DROP-OUT

RELAY. A DROP-OUT RELAY IS A RELAY THAT SENSES A PHONE GOING OFF HOOK, AND

SIGNALS A TAPE RECORDER TO START RECORDING.

ANOTHER TEST TO DO WITH THE PHONES STILL HOOKED UP TO THE OUTSIDE WORLD, ON

HOOK VOLTAGE IS ABOUT 48 VOLTS AND OFF HOOK IS ABOUT 6-10 VOLTS.  ANY OTHER

CONDITIONS MAY MEAN TELEPHONE SURVEILLANCE.

IF YOU USE A WIDE RANGE AUDIO FREQUENCY GENERATOR AND CALL YOUR HOUSE,

APARTMENT, ETC. FROM ANOTHER PHONE AND SWEEP UP AND DOWN THE SPECTRUM, AND YOU

NOTICE THE PHONE ANSWERS ITSELF SOMEWHERE IN THE SWEEP YOU PROBABLY HAVE AN

INFINITY TRANSMITTER ON YOUR LINE.